DELIVERY KIT  /  VC

# Vendor Realization and Shared-Infrastructure Cards

A build track for each hosting stack and its shared foundations.

Kit v1.0 • 16 September 2026 • Aligned to the frozen v1.4 reference architecture

Working kit, not an approved site design or deployed platform. Templates, reference examples and live evidence are separate records.

Use one card per selected realization. These cards state the native engineering decisions and build responsibilities; actual supported versions, settings, credentials and execution evidence must be entered in ET and the schedules.

## Section links

1. Common scope and native implementation contract

2. Nutanix realization card

3. VMware and NSX realization card

4. OpenStack realization card

5. Physical fabric and OOB realization card

6. Shared services and security-edge realization card

7. Support tuple, variations and evidence checklist

All additional process guidance is proposed kit practice. RA/WD references identify baseline-derived architecture; K references identify external mechanism checks. No site values or approval signatures are supplied.

# 1. Common scope and native implementation contract

The common WD fixture is the comparison baseline: tenant-01 and tenant-02, domains D01O/D01R/D02O/D02R, four permanent endpoints, two tenant boundary contexts and explicit shared-service handoffs. A temporary probe is needed for same-domain tests.

Baseline and related records: VND §1  •  WD §2  •  WD §11

| Invariant | Native realization must show |
| --- | --- |
| Independent authority | Tenant administration, domain routing, mandatory policy and management scope are separate and enforced. |
| ZIP-controlled transitions | All required forward/reply paths use qualified boundary functions; native paths do not silently bypass them. |
| Shared-service consumption | Exact clients and endpoints, origin-specific reply, backend entitlement and administrative exclusion. |
| Resource eligibility | Compute, storage, copies and recovery remain inside approved sharing/location boundaries. |
| Lifecycle ownership | Install/commission, allocate, update, adopt, replace, delete and interrupted-operation repair have one owner. |
| Equivalent outcome | Same acceptance semantics and observed evidence, not matching native IDs or identical topology. |

Do not install three stacks just to make the first stack qualify. Prove one controlled realization, then repeat the required comparison and representative image/data recovery on a second target. Treat a composite cross-stack WSD as an explicit additional design.

Common prerequisites: adopted design; approved test scope; actual platform tuple; accepted foundation and required bootstrap; scoped identities; capacity and restore material; current native configuration artifacts.

# 2. Nutanix realization card

Reference mapping: AHV/AOS with protected Prism/Flow administration, separate domain routing contexts, protected endpoint policy and independently owned edge and shared-service integrations.

Baseline and related records: RA §16  •  VND §3  •  WD §8

| Build concern | Specific engineering decision / receipt |
| --- | --- |
| Installation and management | Selected supported cluster/Prism/Flow installation, exact component releases, management path, cluster health and recovery dependencies. |
| Pools and data | Eligible AHV placement, AOS/controller sharing, storage classes, image baseline, encryption/protection and failure headroom. |
| Tenancy and policy | Project/RBAC entitlement distinct from VPC routing; provider-owned security categories/selectors and mandatory effective policy. |
| Domain/edge attachment | Native VPC/subnet/gateway objects and supported external handoff. No-NAT is conditional; shared connected subnet isolation must be proven. |
| Provisioning sequence | Installer-owned P2 foundation → address/attachment reservation → denied domain networks → edge/routes/policy → VM/disks → services and tests. |
| Acceptance / failure | Same-host/cross-host controls, native routing, return symmetry, lost task response, protected data, HA placement and isolated restore. |

The official provider repository publishes compatibility and resource-specific lifecycle notes. Use the exact supported combination and inspect replacement/update behaviour; a v2 resource name does not prove every needed operation. No provider release is selected by this kit. [K05]

External mechanism context: K05 — Official Nutanix Terraform provider repository

Stop when the actual gateway/handoff mechanism, required policy authority or resource lifecycle is unsupported or unproven. Use a reviewed alternative or exclude the capability; do not weaken the boundary to complete the run.

# 3. VMware and NSX realization card

Reference mapping: protected vCenter/ESXi and NSX management, eligible compute/storage pools, independent domain routing and an explicitly isolated upstream path to the required ZIP.

Baseline and related records: RA §17  •  VND §4  •  WD §8

| Build concern | Specific engineering decision / receipt |
| --- | --- |
| Installation and transport | Actual vCenter/ESXi/NSX versions and hardware compatibility; transport nodes/zones, Edge placement, TEP/uplink reachability and management. |
| Routing topology | Map Tier-1, distributed/service routing, Tier-0/VRF or approved alternative, advertisements, native routes and exact boundary next hops. |
| Stateful enforcement and HA | Locate mandatory DFW and boundary functions; choose supported HA mode, state synchronization, return symmetry and failure load. |
| Pools and data | Actual host eligibility/affinity, supported vMotion/restart, datastore/storage policy, images/virtual security devices and backup/key dependencies. |
| Provisioning sequence | P2 compute/storage/NSX transport → scope and isolated upstream capacity → denied segments/gateways → policy → VM/data → service and path checks. |
| Acceptance / failure | No unauthorized native/inter-VRF route; same-host rules, Edge/uplink loss, maintenance placement, support limits and useful restore. |

Technical review item KIT-TN-01: Broadcom KB 442835 describes Tier-0 VRF incompatibility with an active-active stateful parent Tier-0. Reconcile this limitation with the selected release and architecture before choosing HA mode; do not assume every VRF/stateful combination is supported. [K09]

External mechanism context: K09 — Broadcom KB 442835 — Tier-0 VRF and active-active stateful HA

The frozen RA remains unchanged. This kit adds a support check, not a command to change a live gateway. An HA-mode change needs separate disruption, session and recovery analysis.

# 4. OpenStack realization card

Reference mapping: selected OpenStack distribution; protected Keystone/control services; Nova/Placement compute; Neutron with a named backend; Glance/Cinder and separately selected data/protection services.

Baseline and related records: RA §18  •  VND §5  •  WD §8

| Build concern | Specific engineering decision / receipt |
| --- | --- |
| Distribution and control | Service/database/messaging versions, certificates, API policy, installer/upgrade owner and controller recovery. |
| Compute and data | Eligible host aggregates/traits/flavours with actual enforcement; images, boot/drivers, Cinder types/backends, storage credentials and protection. |
| Network backend | Selected ML2/OVN or qualified alternative; controller/gateway/compute roles, mappings, distributed routes and external/provider attachments. |
| Mandatory authority | Provider owns baseline security groups, port security, permitted address pairs and external mutation. Direct tenant editing is a qualified extension. |
| Provisioning sequence | Supported distribution build → scoped project/quotas → denied networks/ports/routers and edge → instances/volumes → services and tests. |
| Acceptance / failure | Effective additive policy, extra NIC/provider routes, metadata/DHCP, gateway/control loss, relocation, storage isolation and stale port/router cleanup. |

Neutron describes security groups as additive allow controls; do not treat several groups as a provider deny hierarchy. OVN distributed routing must be included in path analysis, and Nova placement controls must be configured rather than inferred from an aggregate name. [K06–K08]

External mechanism context: K06 — OpenStack Neutron networking concepts  •  K07 — OpenStack OVN reference architecture  •  K08 — OpenStack Nova host aggregates

Do not compete with Neutron by directly managing the same backend OVN objects. If the selected API/policy combination cannot protect the required baseline, the corresponding delegated capability is not offered.

# 5. Physical fabric and OOB realization card

Reference mapping: provider-owned routed transport, qualified host/appliance attachment, optional EVPN/VXLAN where needed, and separately protected hardware recovery and management.

Baseline and related records: RA §5  •  RA §6  •  NET §1  •  NET §5

| Build concern | Specific engineering decision / receipt |
| --- | --- |
| Inventory and ports | Actual switch/network OS/NIC/optic/firmware compatibility, port map, power/failure grouping and staging identity. |
| Underlay | Link/loopback allocations, permitted peers/ASNs, explicit import/export, ECMP, control-plane protection and maximum-prefix handling. |
| Optional fabric overlay | RD/RT/VNI authority, gateway location, supported route types, BUM/ARP/ND and explicit route-import restrictions. |
| Multihoming | Chosen MLAG or EVPN mechanism, peer/ES dependencies, orphan and split-brain behaviour; no assumed cross-vendor MLAG pair. |
| Platform/edge transport | Owned tunnel endpoint networks, dedicated/qualified handoffs, actual encapsulation and minimum surviving MTU. |
| Commission and recover | Supported network configuration owner, safe staged change, snapshots, constrained management, fault/recovery observations and accepted G1 receipt. |

Use a site-specific network-OS annex after hardware selection. A Dell OS10 or other switch implementation must supply its actual provider or native configuration coverage and release-specific commands. The kit does not assert an unverified universal Terraform switch provider.

Physical placement and cabling execution use approved facility/manufacturer procedures and qualified personnel. Do not infer physical independence from two logical links terminating on the same failure dependency.

# 6. Shared services and security-edge realization card

These resources are required integrations, not incidental dependencies automatically supplied by the selected hypervisor provider.

Baseline and related records: RA §9  •  SVC §1  •  SVC §5  •  WD §7

| Service track | Native design and implementation receipt |
| --- | --- |
| Security edge | EC/SE or equivalent contexts, exactly scoped routes/interfaces/policy, joint boundary authority, required inspection, session behaviour, HA and management. |
| IPAM/name/time | Delegated address authority, reservations/leases, DNS forward/reverse and approved resolvers including needed TCP/UDP, time profile and ownership. |
| Identity and trust | Enterprise roles/federation, workload and runner identities, certificate lifecycle, key-use/admin/recovery scope and independent bootstrap. |
| Storage/data | Owned virtual disks, file/object endpoint authorization, shared backend implications, capacity/QoS, snapshots/replication and sanitization. |
| Backup/recovery | Capture orchestration versus transfer path, repository protection, catalogue/keys, retained-copy ownership and isolated useful restore. |
| Telemetry and supply | Attributable event collection, buffering/loss handling, authorized search/retention, trusted images/artifacts and publication authority. |

Each track publishes a handoff containing actual identity, permitted use, capacity/limits, support version, lifecycle owner, current acceptance restrictions and evidence. Sharing a reference does not require sharing the producer’s privileged credentials or full Terraform state.

SVC-REF is an illustrative shared-service domain, not permission for all provider traffic. Service-side return routing, source validation and backend entitlement remain part of qualification. Record each service’s compromise and outage dependency.

# 7. Support tuple, variations and evidence checklist

Complete this review for each selected platform and all dependencies used by the offered service. A moving “latest” documentation page is context; retain the exact supported release evidence for the actual installation.

Baseline and related records: VND §7  •  ET §8  •  IK §4

| Decision/check | Evidence and disposition |
| --- | --- |
| Native resource mapping | Reference IDs → actual platform objects, interfaces, hosts, data objects and owners. |
| Compatibility and entitlements | Exact tuple plus official support matrix/release notes and actual enabled features. |
| Operation coverage | Per-resource observe/create/update/adopt/replace/delete and interrupted-task discovery with a supported alternative where needed. |
| Control/function coverage | Every required boundary, identity, data, placement and recovery function has an enforcement location and an observation. |
| Limits and failures | Measured service capacity, relevant HA/partition behaviours and forbidden alternative paths. |
| Variations and residual gaps | Changed design requires appropriate approval and new assertions; no automatic portability claim. |
| Readiness | Qualification scoped by class/family; required operational recovery evidence before production; ongoing requalification triggers. |

Use source IDs K05–K09 as current research entry points and the frozen RA/VND as architecture lineage. Source discovery does not approve a product combination. A feature gap should become a documented alternative, a limited service offer or a blocking issue.

These cards are implementation specifications and checklists. Actual modules, native installation/configuration artifacts, environment values and live evidence are separate required outputs—not represented as already delivered.